AC CONTROLLER - 6

Name : B.Tharun Kumar Reddy

Reg No:192565015

➔ Source Code:

#define TEMP A0 #define FAN 8 volatile byte pwm = 0; volatile byte count = 0;

// Timer1 Compare Match Interrupt

ISR(TIMER1_COMPA_vect)

{   count++;   if (count >= 100)

{     count = 0;

}

digitalWrite(FAN, count < pwm);

} void setup() {

Serial.begin(9600);   pinMode(FAN, OUTPUT);

// Timer1 configuration

TCCR1A = 0;

TCCR1B = (1 << WGM12) | (1 << CS11);

OCR1A = 199;   TIMSK1 = (1 << OCIE1A);

sei(); }

void loop() {   int adc = analogRead(TEMP);

// Convert ADC value into temperature (LM35 sensor)   float temp = adc * 500.0 / 1023.0;   if (temp < 25)

{

pwm = 0;

}   else if (temp < 30)

{

pwm = 30;

}

else if (temp < 35)

{

pwm = 60;

}   else

{

pwm = 90;

}

Serial.print("Temperature: ");

Serial.print(temp, 1);

Serial.print(" C  PWM: ");

Serial.print(pwm);   Serial.println("%");   delay(1000);

}

Reflections:

- Learned how to interface the LM35 temperature sensor with Arduino Uno to measure temperature.

- Understood ADC conversion and the process of converting analog sensor readings into temperature values.

- Gained practical knowledge of Timer1 configuration and Compare Match Interrupts.

- Learned to generate software-based PWM signals to control fan speed according to temperature variations.

- Understood how embedded systems use real-time sensor data for automatic temperature monitoring and fan control.